ZAVA CHEMICALS SDN BHD / PT ZAVA KIMIA INDONESIA

HAZARD IDENTIFICATION, RISK ASSESSMENT
& RISK CONTROL (HIRARC)

Cilegon Specialty Chemicals Plant -- Banten, Indonesia
Document No: ZC-HSE-HIRARC-2025-001 | Revision 3 | March 2025

REGULATORY STATUS: 3 ENFORCEMENT NOTICES ACTIVE (Jan-Mar 2025)
Prepared by: Cilegon EHS Manager | Reviewed: Group EHS Director | Approved: Zava Chemicals MD

Section 1: Enforcement Notices -- Status & Response

ENFORCEMENT NOTICE #1 -- Effluent Discharge Breach (January 2025)

Notice Details: Issued by: BPOM (Badan Pengawas Obat dan Makanan) | Date: 15 January 2025 | Reference: BPOM/EN/2025/CIL-001

Finding: Effluent from Process Line B (chlorinated paraffin production) found to exceed permitted Chemical Oxygen Demand (COD) limits. Sampling conducted 8 January 2025 by BPOM inspector. COD measured at 2,840 mg/L against permitted limit of 300 mg/L. Effluent Treatment Plant (ETP) bypass valve found partially open.

Root Cause: Night shift operator bypassed ETP due to ETP pump failure (pump P-204A out of service since Dec 2024). Maintenance requisition submitted but parts not procured in time.

Immediate Action (15 Jan 2025): ETP bypass valve sealed; production on Line B suspended; emergency ETP pump procurement initiated

Corrective Action: ETP pump P-204A replaced 28 Jan 2025; full ETP functional test completed 2 Feb 2025; Line B production restarted under enhanced monitoring

Preventive Action: ETP bypass valve fitted with solenoid lock requiring HSE Manager authorisation; daily effluent monitoring increased from weekly

Status: Corrective action verified by BPOM re-inspection on 15 Feb 2025. Notice formally closed 20 Feb 2025.

ENFORCEMENT NOTICE #2 -- Chemical Storage Labelling Non-Compliance (February 2025)

Notice Details: Issued by: BPOM | Date: 22 February 2025 | Reference: BPOM/EN/2025/CIL-002

Finding: During unannounced inspection of Chemical Storage Warehouse B (intermediate chemicals), 18 drums of Dimethyl Sulfoxide (DMSO) found without GHS-compliant labels. Labels had degraded due to chemical exposure. 6 drums of Chlorinated Paraffin found with English-only labels (Indonesian language required by PP 74/2001).

Root Cause: Label inspection not included in monthly warehouse audit checklist. Chemical resistance of label material inadequate for DMSO storage conditions.

Immediate Action (22 Feb 2025): All 180 chemical storage drums re-labelled with GHS-compliant bilingual labels within 72 hours

Corrective Action: Procured chemically-resistant polypropylene label stock; updated warehouse audit checklist to include label integrity check

Preventive Action: Monthly GHS labelling audit added to SOP-HSE-012; vendor qualified for emergency label reprinting within 24 hours

Status: OPEN -- BPOM re-inspection scheduled 30 April 2025. Internal verification completed March 2025.

ENFORCEMENT NOTICE #3 -- PPE Non-Compliance (March 2025)

Notice Details: Issued by: Disnaker Banten (Banten Provincial Labour Office) | Date: 8 March 2025 | Reference: DISNAKER/EN/2025/BTN-041

Finding: During routine inspection of Process Area C (fatty acid distillation), 7 workers observed operating without full chemical-resistant face shields. Workers were wearing safety glasses only. Process Area C handles concentrated sulphuric acid and elevated temperature condensates. Additionally, 3 workers found without chemical-resistant gloves during equipment cleaning operation.

Root Cause: Inconsistent enforcement of PPE requirements by shift supervisors; PPE wearing non-negotiable rule not reinforced at pre-shift briefings

Immediate Action (8 Mar 2025): Work stopped in Process Area C; PPE compliance verified for all 28 workers before restart

Corrective Action: Re-training for all 3 shift supervisors on PPE enforcement; PPE compliance check added to pre-shift toolbox talk

Preventive Action: CCTV coverage extended to Process Area C for PPE monitoring; PPE compliance rate added to shift supervisor KPIs

Status: OPEN -- Disnaker re-inspection scheduled 15 May 2025. TRIR increased to 2.8 following related near-miss report.

Section 2: HIRARC Matrix -- Chemical Process Hazards

Risk matrix uses DOSH Malaysia HIRARC methodology (Likelihood 1-5 x Severity 1-5 = Risk Rating 1-25). Risk levels: Low 1-4, Medium 5-9, High 10-16, Extreme 17-25. Control hierarchy: Elimination > Substitution > Engineering > Administrative > PPE.

| Hazard ID | Activity | Hazard | Potential Consequence | L | S | RR | Controls Required |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H-001 | H2SO4 handling | Concentrated sulfuric acid splash | Chemical burns, blindness, fatality | 3 | 5 | 15 HIGH | Full face shield, acid suit, emergency shower within 10m, buddy system, SOP-HSE-004 |
| H-002 | DMSO distillation | DMSO vapour inhalation at high temp | CNS effects, dizziness, headache | 3 | 3 | 9 MED | LEV (Local Exhaust Ventilation), half-face respirator (organic vapour cartridge), LEL monitoring |
| H-003 | Chlorinated paraffin | Skin/eye contact with molten CP | Burns, skin sensitisation | 2 | 3 | 6 MED | Chemical gloves (nitrile 0.4mm min), splash goggles, thermal suit, SOP-HSE-009 |
| H-004 | ETP operations | H2S generation in effluent tank | Toxic gas poisoning, fatality | 2 | 5 | 10 HIGH | Confined space entry permit, H2S monitor (alarm at 1 ppm), SCBA on standby, 2-man rule |
| H-005 | Process Line B startup | Pressure vessel rapid depressure | Explosion, projectile, fatality | 1 | 5 | 5 MED | Pre-startup safety review (PSSR), PSV tested annually, blast shield, exclusion zone 25m |
| H-006 | Fatty acid loading | Hot fatty acid transfer (180 degC) | Thermal burns, fire | 3 | 4 | 12 HIGH | Heat-resistant suit, face shield, thermal gloves, loading arm interlock, firewater hose on standby |
| H-007 | Chemical storage | Incompatible chemical storage | Fire, explosion, toxic cloud | 2 | 5 | 10 HIGH | Segregation per NFPA 704 matrix; secondary containment 110% drum capacity; HazMat store SOP |
| H-008 | Laboratory sampling | Exposure to process intermediates | Various chemical exposure | 3 | 3 | 9 MED | Fume hood for all volatile sampling, PPE per MSDS, buddy system, first aid kit in lab |
| H-009 | Forklift drum movement | Drum drop/forklift collision | Spill, fire, crush injury | 3 | 3 | 9 MED | Forklift certification, pedestrian exclusion zones, drum clamp attachment, speed limit 8 km/hr |
| H-010 | Night shift operations | Reduced supervision, fatigue | Increased error rate, incident | 4 | 4 | 16 HIGH | Shift supervisor patrol every 2 hrs, fatigue management policy, CCTV monitoring, buddy check |
| H-011 | Maintenance hot work | Welding/cutting near chemical residues | Fire, explosion | 2 | 5 | 10 HIGH | Hot work permit (PTW), area gas-free certificate, fire watch, firewater on standby, 9m exclusion |
| H-012 | Emergency response | Fire/explosion during emergency | Secondary casualties | 2 | 5 | 10 HIGH | Emergency Response Plan (ERP-CIL-001), monthly fire drill, HAZMAT response team, BPBD notification protocol |

Section 3: Chemical Hazard Register (24 Hazardous Chemicals)

| No | Chemical Name | CAS No | GHS Hazard Class | Flash Point | IDLH (ppm) | Emergency Action |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sulfuric Acid (conc.) | 7664-93-9 | Corrosive, Oxidising | Non-flammable | N/A | Dilute with water; neutralise with sodium bicarbonate; eye wash 15 min |
| 2 | Dimethyl Sulfoxide (DMSO) | 67-68-5 | Flammable, Skin penetrant | 95 degC | N/A | Remove to fresh air; skin flush water 20 min; dermal absorption risk |
| 3 | Chlorinated Paraffin (C14-17) | 85535-85-9 | Aquatic toxic, PBT suspected | >200 degC | N/A | Prevent waterway entry; HAZMAT containment; BPOM notification within 2 hrs |
| 4 | Fatty Acid (Oleic Acid) | 112-80-1 | Flammable liquid Gr 3 | 189 degC | N/A | Foam or dry powder; hot fatty acid > 100 degC splash = thermal burns |
| 5 | Glycerine (Refined) | 56-81-5 | Combustible liquid | 177 degC | N/A | Water fog; minor skin/eye irritant only |
| 6 | Fatty Alcohol (C16/C18) | 36653-82-4 | Flammable solid (molten) | >200 degC | N/A | Dry sand or CO2; avoid water on molten material |
| 7 | Methanol (process solvent) | 67-56-1 | Flammable, Toxic | 11 degC | 200 ppm | Alcohol-resistant foam; CNS/optic nerve toxin; SCBA required |
| 8 | Sodium Hydroxide (50%) | 1310-73-2 | Corrosive | Non-flammable | N/A | Water flush 20 min; do not neutralise on skin; notify medical |
| 9 | Sodium Lauryl Sulfate (SLS) | 151-21-3 | Irritant | N/A | N/A | Eye/skin flush; non-toxic at workplace levels |
| 10 | Isostearic Acid | 2724-58-5 | Flammable liquid Gr 3 | 170 degC | N/A | Dry chemical or CO2; mild skin irritant |
| 11 | Caprylic Acid | 124-07-2 | Flammable, Irritant | 110 degC | N/A | CO2 or dry powder; strong odour; mild corrosive |
| 12 | Capric Acid (Decanoic Acid) | 334-48-5 | Flammable solid | 112 degC | N/A | Water fog or CO2; irritant to skin, eyes |
| 13 | Epoxidised Soybean Oil (ESBO) | 8013-07-8 | Combustible | 260 degC | N/A | Dry chemical; low acute toxicity |
| 14 | Hydrogen Peroxide (35%) | 7722-84-1 | Oxidising, Corrosive | N/A (oxidiser) | N/A | Water dilution; do not contain spill -- ventilate; reactive with organics |
| 15 | Phosphoric Acid (75%) | 7664-38-2 | Corrosive | Non-flammable | N/A | Water flush; neutralise with calcium carbonate; do not combine with H2SO4 |
| 16 | Nitrogen Gas (liquid/compressed) | 7727-37-9 | Asphyxiant | Non-flammable | N/A -- asphyxiation | Confined space permit; O2 monitor; SCBA; evacuate to open air |
| 17 | Propylene Glycol | 57-55-6 | Flammable liquid Gr 3 | 99 degC | N/A | Water or CO2; low toxicity; large spill fire risk |
| 18 | Toluene | 108-88-3 | Flammable, CNS toxin | 4 degC | 500 ppm | LEL/O2 monitor; SCBA; foam suppression; notify fire brigade > 50L spill |
| 19 | Caustic Potash (KOH 50%) | 1310-58-3 | Corrosive | Non-flammable | N/A | Immediate water flush eyes/skin; medical attention; do not neutralise on skin |
| 20 | Trimethylolpropane (TMP) | 77-99-6 | Combustible solid | 150 degC | N/A | Water fog; low acute toxicity; dust explosion risk when dry |
| 21 | Diethyl Sulfate | 64-67-5 | Toxic, Carcinogen | 104 degC | N/A | SCBA; PPE Lv B minimum; BPOM immediate notification; carcinogen -- decontaminate fully |
| 22 | Monochloroacetic Acid | 79-11-8 | Toxic, Corrosive | 126 degC | N/A | SCBA; full chemical suit; toxic by skin absorption; medical surveillance required |
| 23 | Formic Acid (85%) | 64-18-6 | Flammable, Corrosive | 69 degC | N/A | Water dilution; CO2; corrosive vapour -- LEV required; eye wash 15 min |
| 24 | Ammonia (28% solution) | 1336-21-6 | Toxic gas in solution | N/A (flammable gas) | 300 ppm | Water fog to suppress vapour; SCBA; upwind evacuation; notify BPBD |

Section 4: Emergency Response Procedures

4.1 Emergency Response Organisation

Incident Commander: Plant Manager (or deputy in absence)

Fire & Rescue Team: 12 trained HAZMAT first responders (NIOSH certified)

Medical First Responder: 2 occupational health nurses; on-call plant medical officer

Emergency Contact: BPBD Cilegon (+62-254-391-xxx); BPOM Banten (+62-254-398-xxx); PLN Cilegon; RSPAD military hospital

Muster Points: 3 designated muster points (MP-01 main gate, MP-02 north perimeter, MP-03 east jetty)

Emergency drills: Full evacuation quarterly; HAZMAT spill drill bi-annually; fire drill monthly

4.2 Spill Response Protocol

For any chemical spill >10 litres or any quantity of Diethyl Sulfate, Monochloroacetic Acid, or Ammonia:

1. Activate emergency alarm (PA system code RED SPILL)

2. Evacuate non-essential personnel from spill zone (minimum 25m radius)

3. Don appropriate PPE before approaching (refer Chemical Hazard Register for PPE level)

4. Isolate source if safe to do so (valve closure, drum upright)

5. Contain spill with HAZMAT absorbent (vermiculite for acids; clay for hydrocarbons)

6. Notify Plant EHS Manager and Incident Commander immediately

7. BPOM notification within 2 hours for any spill reaching drainage channels

8. Decontaminate affected personnel; medical assessment mandatory

9. Document on HSE Incident Report Form HSE-IR-001 within 24 hours

10. Root cause analysis and corrective action within 5 working days

4.3 KPIs & Safety Dashboard

FY2024 Safety KPIs: Group TRIR: 0.84 | Cilegon TRIR: 1.84 (target <0.8) | LTIR: 0.42 | Near Misses Reported: 124 | Safety Observations: 1,840

Q1 FY2025 Cilegon: TRIR spiked 2.8 (Feb 2025 -- PPE incident) | 3 enforcement notices | 2 production lines suspended | EHS Manager position VACANT -- hiring in progress

Zava Chemicals | Cilegon Plant HIRARC | Doc No: ZC-HSE-HIRARC-2025-001 | Rev 3 | CONFIDENTIAL